|  | DOING PHYSICS WITH MATLAB MECHANICS ELECTRIC MOTOR ANIMATION Ian Cooper School of Physics, University of Sydney ian.cooper@sydney.edu.au DOWNLOAD DIRECTORY FOR MATLAB SCRIPTS mscript missing – but the animation shows how Matlab can be used to create animated drawings. An electric motor is an electromechanical device that converts electrical energy to mechanical energy. When a current flows through the armature (coil windings) in the magnet field it will experience a torque. However, the coil can only rotate continually in one direction if the direction of the current is reversed for each half turn of the coil to keep the direction of the toque on the current loop constant. The basic components of the of a DC motor are show in figure (1). Fig. 1. The basic parts of a DC motor. The motion of the coil ABCD in the magnetic field is animated in figure (2). It shows the view of the coil looking along the +y direction. Only the sides AB and CD experience a torque for the rotation of the coil. The magnitude of this torque changes as the coil rotates since the angle between the directions of the magnetic field and the plane of the coil continually changes. The current in arm AB is shown and how its direction is reversed each half cycle so that the direction of the torque is constant. The direction of the magnetic force on a current element is at right angle to the directions for the magnetic field and the current. The directions can be determined by the right hand palm rule. Fig. 2. Animation of the armature of a DC motor turning in the magnetic field. |  |
| --- | --- | --- |